Basic Configuration of a Wireless Router

Note: Prepare a report, including some screenshots of the tasks you complete.

The router is a TP-Link TD-W8960N 300Mbps wireless N ADSL2+ router.

The router ships with a default IP address of 192.168.1.1, and a login of username: admin, password: admin.

There is no DHCP pool configured initially, so you can connect through one of the switch ports, assign a static IP address of 192.168.1.10/24 to the PC, and connect to the router configuration by entering 192.168.1.1 into the browser address bar. Log in. You will see something like the following:

On the left-hand pane, there are menu options. Select the Quick Setup option.

You can skip the WAN Configuration, as we don’t have a DSL connection.  You will be brought to the wireless configuration where you can enable wireless, change the SSID, and set up the authentication for the network. Choose WPA2-PSK, and choose a password.

Advanced Setup

The advanced setup looks as follows:

Click on LAN option. Enable DHCP Server, and set up a DHCP pool from 192.168.1.100 to 192.168.1.200.

Connect to the wireless network using a laptop/phone.

When you have connected, check your IP settings. They should look something like this:

Investigate the different Security options on the LAN interface.

Click on the Wireless option. You will see something like this:

Note how you can Disable SSID Broadcast. Test this.

Click on the Security option and investigate what changes you can make:

Note: It is recommended that you disable WPS due to security concerns.

Setup a MAC filter and check that it works:

Click on the Advanced option. Note that the Channel is Auto, by default. See how you can set the channel from 1 – 13 manually.

Observe how you can change the Mode, Bandwidth, and Beacon Interval.

Have a look at the Diagnostics and Management options. Investigate what can be done here.

Exercise:

Prepare a report, including some screenshots of the tasks you completed.

Setup:

Security –Mac address:

Security –Wireless: